| Search Words (required) | Volvo, Volvo trucks, Volvo D13, turbo compounding, boost, horsepower, torque, big rig, D11, D16, D13, exceptional efficiency |
| --- | --- |
| Reuse Permissions (required) | insert value |
| Writer or Editor (required) | Brian Lohnes |
| Photographer | Courtesy of the manufacturers |
| Illustrator | insert value |
| Web Head (required) 60 character max recommended | Volvo’s New D13 With Turbo Compounding Pushes Technology and Efficiency In Trucking |
| Related Web Articles | insert value |

| Article Summary for Google 150 character max | Volvo trucks has introduced a new variant of their D13 engine that is more powerful and efficient than ever before. New transmission and exhaust DPF introduced as well. |
| --- | --- |

DSLP-160800-POW

Volvo D13TC

The latest offering from Volvo ups efficiency and power with amazing technology

By Brian Lohnes

Photos courtesy of Volvo trucks

When it comes to the world of trucking, power and efficiency are king. Sure diesel prices are favorable now but that doesn’t mean they always will be and today’s truck buyers want the grunt to pull their cargo and they also want those same powerful engines to sip fuel. Volvo Trucks heard those calls loud and clear and recently debuted a new version of their venerable D13 engine to meet the rising expectations of their customers. The D13TC (Turbo Compound) will be offered in 2017 model year rigs and the company has claimed fuel efficiency increases of up to 6.5% as well as power ratings that top 450 hp and 1,850 lb-ft of torque. A one point bump in compression helped to add power but an amazing new addition to the engine really changes the game. It’s called turbo compounding and it may blow your mind.

The turbo compounding system that Volvo uses on the D13TC is very interesting. This is not a pair of turbochargers feeding each other. Instead there is an additional turbine wheel in the flow path of the exhaust and that second spinning wheel is connected to a fluid coupling.  The fluid coupling turns a system of reduction gears until the system is ultimately exerting force on the crankshaft via the flywheel. Volvo claims that this turbo compounding system can add as much as 50 additional horsepower to the engine. In simpler terms, imagine that you are pedaling a bicycle and in addition to your pedaling there’s a hand helping to shove the rear wheel along as well. That’s essentially what this system is doing in the D13TC. The technology has been used in european Volvos for years and now it comes here to the USA for the 2017 model year. Volvo recommends the D13TC engine to customers who spend most of their time on the open road and not for truck that live the city life primarily. Another cool advancement in the universe of diesel technology!

SPECIFICATIONS

Engine: Volvo D13TC

Displacement: 13L 780ci

Engine layout: Inline six cylinder

Power: 455 hp @ 1,300 rpm

Torque: 1,850 lb-ft @ 900 rpm

Bore x Stroke: 5.16 x 6.22 inches (131 x 158 mm)

Compression ratio: 17.0:1

Fuel delivery: Common Rail injection

Induction: Turbocharger with turbo compounding

Aftercooling: Air-to-air intercooler

Cooling: Pressurized, closed-circuit liquid system

Oiling: Wet Sump system with 38 quart capacity

Weight: 2,605 pounds (1,182 kg)

PHOTOS

001-Volvo-D13TC-turbo-side-view

The Volvo D13TC is one of the most technologically advanced engines ever used in North American trucking. With 455 hp on tap and 1,850 lb-ft of torque at less than 1,000rpm it is designed to be both powerful and very efficient. Truckers demand both and they get it here!

002-Volvo-D13TC-intake-side-view

These big engines have come a long way from the smoking two-strokes of yesteryear!While you may not be able to fix them with a hammer and a hatchet anymore, engines like the Volvo D13TC outlast, outperform, and outwork the old timers by a wide margin. These are marvels of modern technology.

SOURCES:

Volvo Trucks

http://www.volvotrucks.com